# SMART VEHICLE BLACKBOX WITH ACCIDENT DETECTION AND EMERGENCY RESPONSE

## Detailed Project Report

## Abstract

Road accidents often lead to delayed emergency response, increasing the risk of fatalities and severe injuries. This project presents a Smart Vehicle Blackbox system designed to improve post-accident response through automated accident detection, location tracking, and emergency alert generation.

The system uses an Arduino UNO as the central controller, an MPU6050 sensor for impact and rollover detection, a NEO-6M GPS module for obtaining accident coordinates, and a serial/GSM alert mechanism for notifying emergency contacts. Upon detecting an accident, the system immediately triggers an emergency alert, then sends a follow-up location update when GPS coordinates are available. The system then enters a lock state to preserve event information until manually reset.

This prototype acts as a low-cost intelligent automotive blackbox aimed at improving safety, reducing response time, and supporting post-accident analysis.

# 1. Introduction

A conventional aircraft blackbox stores flight information for analysis after incidents. Inspired by this concept, this project implements a similar blackbox idea for automobiles.

The developed system continuously monitors vehicle motion using an inertial sensor and detects:

- Sudden impacts

- Vehicle rollover

- Abnormal acceleration events

When an accident is detected:

- Immediate SOS alert is generated.

- GPS coordinates are acquired.

- Follow-up location notification is sent.

- System enters blackbox lock mode.

This reduces dependency on manual emergency reporting.

# 2. Problem Statement

In many accidents:

- Victims may be unconscious.

- Emergency services are not informed quickly.

- Accident location may be unknown.

- Post-crash data is often unavailable.

A low-cost automatic accident reporting system is needed.

# 3. Objectives

The objectives of this project are:

- Detect vehicle accidents automatically.

- Detect rollover events.

- Send immediate emergency alert.

- Provide accident location using GPS.

- Preserve accident event data.

- Develop a compact, affordable prototype.

# 4. System Overview

## Main Functional Blocks

The system consists of:

### Input Section

- MPU6050 motion sensor

- GPS satellite signals

### Processing Section

- Arduino UNO

- Accident detection algorithm

### Output Section

- Emergency alerts

- Location update notification

- Blackbox lock mode

# 5. Components Used

## Hardware Components

| Component | Purpose |
| --- | --- |
| Arduino UNO | Main controller |
| MPU6050 | Accident and rollover detection |
| NEO-6M GPS Module | Location tracking |
| Breadboard | Prototyping |
| Jumper Wires | Connections |
| Power Supply | System power |

## 5.1 Arduino UNO

Acts as the brain of the system.

Functions:

- Reads sensor values

- Processes accident logic

- Generates alerts

- Controls system operation

## 5.2 MPU6050 Sensor

The MPU6050 is a 6-axis IMU containing:

- 3-axis Accelerometer

- 3-axis Gyroscope

Used for:

- Sudden impact sensing

- Rollover detection

- Motion anomaly detection

## 5.3 NEO-6M GPS Module

Provides:

- Latitude

- Longitude

- Satellite lock

- Vehicle position

Used to send accident coordinates.

# 6. Working Principle

## Step 1 — Continuous Monitoring

The MPU6050 continuously measures:

- X-axis acceleration

- Y-axis acceleration

- Z-axis acceleration

## Step 2 — Accident Detection

Acceleration magnitude is calculated:

A = √(Ax² + Ay² + Az²)

If:

- Magnitude > Threshold OR

- Tilt exceeds threshold

Then accident is declared.

Thresholds used:

- Acceleration magnitude > 25000

- |Ax| > 18000

- |Ay| > 18000

## Step 3 — Immediate Alert

As soon as accident occurs:

Emergency notification is triggered instantly.

Example:

“Accident detected. SOS sent. GPS acquiring…”

## Step 4 — GPS Follow-Up Alert

After GPS lock:

Coordinates are sent.

Example:

Latitude: 17.568181
Longitude: 78.431915

Maps link generated:

https://maps.google.com/?q=17.568181,78.431915

## Step 5 — Blackbox Lock Mode

System halts after accident.

Purpose:

- Prevent repeated false triggers

- Preserve event state

- Mimic blackbox behavior

Manual reset required.

# 7. Block Diagram

+----------------+
             |   MPU6050 IMU   |
             +--------+--------+
                      |
                      |
              Motion Data
                      |
              +-------v-------+
              | Arduino UNO   |
              | Accident Logic|
              +---+-------+---+
                  |       |
                  |       |
         +--------+       +---------+
         |                          |
  +------v------+            +------v------+
  | NEO-6M GPS  |            | Alert Output |
  +-------------+            +-------------+

# 8. Circuit Connections

## MPU6050 to Arduino

| MPU6050 | Arduino |
| --- | --- |
| VCC | 5V |
| GND | GND |
| SDA | A4 |
| SCL | A5 |

## GPS Module to Arduino

| GPS | Arduino |
| --- | --- |
| VCC | 5V |
| GND | GND |
| TX | D4 |
| RX | D3 |

## GSM Module to Arduino

| GSM | Arduino |
| --- | --- |
| VCC | 3.7V (Battery +) |
| GND | Common ground |
| TX RX | D10 D11 (Via voltage divider) |

# 9. Software Algorithm

## Accident Detection Algorithm

- Read accelerometer values.

- Compute magnitude.

- Compare against threshold.

- If accident:

  - Trigger immediate alert

  - Wait for GPS

  - Send location update

  - Lock system

## Flowchart

Start
 |
Initialize Sensors
 |
Monitor Motion
 |
Accident Detected?
 |----No----> Continue Monitoring
 |
Yes
 |
Send Emergency Alert
 |
Acquire GPS Coordinates
 |
Send Location Update
 |
Lock System
 |
End

# 10. Features Implemented

## Accident Detection

Using acceleration magnitude and rollover threshold.

## Rollover Detection

Detects abnormal vehicle orientation.

## Two-Stage Alerting

Stage 1: Immediate emergency notification.

Stage 2: Location update after GPS lock.

## Blackbox Locking

System halts after event.

# 11. Code Logic Highlights

## Acceleration Magnitude Logic

Uses sensor fusion:

- Impact spike detection

- Tilt detection

This reduces false alarms.

## Two-Stage Emergency Protocol

Unlike waiting for GPS before alerting, this project:

- Alerts immediately

- Updates location later

Faster response.

# 12. Results

## Observed Results

System successfully:

- Detected accident simulations

- Detected rollover events

- Obtained GPS lock

- Generated emergency alerts

- Produced live location coordinates

- Entered blackbox lock mode

## Sample Output

ACCIDENT DETECTED

EMERGENCY ALERT SENT
SOS Sent
GPS Acquiring...

LOCATION UPDATE SENT
Coordinates:
17.568181,78.431915

System Locked
Manual Reset Required

# 13. Limitations

- GPS requires open sky for lock.

- Thresholds may need calibration for different vehicles.